STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ each

UNIT OF PACK (UPK): 1 (one) each

- Purpose of Equipment:

  - To move palletized cargo from ground level to elevated storage areas and for narrow aisle applications.

- Generalized Operating Specifications

  - Dimensions:

    - 72 inches long, 52 inches wide. Dimensional sizes, such as length, width are not critical to this equipment, it serves as a general guide.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 12 years or more with basic maintenance.

    - Maximum operational load rated capacity of 3500 pounds.

    - Power steering- automotive.

    - Poly load wheels and rubber drive wheels.

    - 48” high load backrest extension.

    - Travel speed limit switch.

# Fork length is 42 inches.

# Collapsed mast unit; height not to exceed 95 inches.

# The reach provides for a 24 inch extension.

# Standard elevated fork height to be 212 inches (Triplex) at full extension.

    - Cold storage to -20 degrees for all OCONUS shipments.

    - Wet charged battery provided for CONUS shipment and dry charged battery for OCONUS shipment.

# 24 Volt system battery (875 ampere hour capacity).

    - Battery Connector-SB350 Anderson Red (Position B- 16inch lead).

    - Horn, key switch, hour meter, and discharge indicator combined.

    - Fire Extinguisher.

    - Back up alarm.

    - Side-shift.

    - Dual headlights.

    - Strobe warning light.

  - Networking / Communications that shall be provided:  N / A

- Operating Options:

  - Standard fork tine length is 42 inches.  Optional fork lengths are:                        ____ 36 inches, ____ 48 inches.

  - ____ Charger required (unless this line is checked, DO NOT SEND charger).

  - ____ Cold storage option, -20 degrees cold storage package.

- Electrical Requirements:  Optional battery charger (CED 1R01), standard CONUS electrical requirement for charger is to be 208-480V / 60 Hz / 3 Ph (not to exceed 20 amps at 480V).  Charger to be factory set to 480 volts with the capability for adjustment at the store by an electrician as needed.

- Color Requirements:  Bright yellow.

- Maintenance Requirements:

  - Sustainability:  Continued support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Requirements:

  - Industry Standards: Underwriters Laboratory (UL) listed, or equivalent.

  - Contractor Requirements:

    - Must be shipped ready to use.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:

# Heavy duty brake system that is automatically applied when the operator leaves the vehicle.

# Combination sensing switch and brake peddle switch that requires the operator to be completely in the operator’s compartment before the truck becomes operational.

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class: 3930

- Special Considerations For Installation/De-installation Of Equipment: (NOTICE/WARNING)

  - Contractor:  N / A

  - Store:  N / A

# Basis of Allocation:

# K-1 to K - 5 one each, quantity in excess of one each is a one for one substitution for a 1R06 unit, not an additional allocation.

- Recommended Source(s) or equal:

  - DOD, Defense Logistics Agency (DLA) contract is DeCA’s source for MHE.

  - Yale Model No. NR035AEN24TE095

- Store Request for Specific Model to Match Existing Equipment: N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Have unit added to the MHE/Baler maintenance service contract.

    - It is normal for the battery to be replaced up to two times during the life cycle of this equipment.