# Purpose of this Equipment:  To provide stores with the ability to power wash carts, walls, sidewalks and other equipment that requires pressurized cleaning.

- Federal Supply Class:  4940

- General Operating Specifications:

  - Dimensions:

    - Dimensional sizes, such as width, depth, and height are not critical to this equipment.

  - Salient Characteristics that shall be provided:

    - Adjustable operating pressure with a minimum of 1000 PSI

    - Minimum 30 feet of high pressure hose

    - Electrically powered pump motor

    - Triggered spray wand

    - Minimum 1HP electric motor

    - Integral ground fault circuit interrupt (GFCI)

    - Minimum of one chemical dispenser / injector (inline or suction)

    - Pump handles water and solutions at temperatures up to 160 degrees Fahrenheit

    - Wheeled, portable model

# Industry Standards / Requirement(s):

    - Industry Standards: Underwriters Laboratory (UL) listed, or equivalent and CETL approved to UL 1776 standards. Conformité Européene (CE) is not acceptable for a standard.

    - Warranty period shall be for (1) year from date of delivery against defective material and workmanship and shall include parts, service and labor.

  - Information Technology Requirements:  N / A

# Color Requirement(s):  N / A

- Electrical Requirements:

# _____ Standard U.S.A. electrical specifications:  120 volts, 60 Hertz, 1 phase.

# Minimum 20 foot cord with a NEMA Standard plug, or a manufacturer’s equivalent to comply with the equipment plug in requirements.

# Average amp draw not to exceed 15 amps.

# NOTE: the operating range for 120 volts is 110 to 120 volts; the operating range for 220 volts is 208 to 230 volts.

# Standard Contractor Requirements:

  - Must be shipped ready to plug in, and use

  - Information to be provided by the Contractor to the commissary at the time of delivery:

    - Point of Contact for Service

    - Installation and Operating Instructions

    - Parts List

  - Maintenance Sustainability Requirements: Continued parts support is required for the projected life cycle of the equipment plus 5 years.

# Special Coordinating / Safety Instructions:  N / A

STORE ORDER REQUEST

STORE NAME: __________________________    DODAAC: ____________________

QUANTITY: _______