# Purpose of this Equipment: To provide stores with the capability to change lights, hang banners and perform minor clean up.

- Federal Supply Class: 3930

- General Operating Specifications:

  - Dimensions:

    - 54 inches long, 32 inches wide, 78 inches high. Dimensional sizes, such as length, width, and height may vary no more than 3 inches in any direction.

  - Salient Characteristics that shall be provided:

    - Minimum working height of 25 feet.

    - Minimum lift capacity of 350 pounds.

    - Non-marking solid tires.

    - Self-propelled.

    - Auxiliary platform controls.

    - Maintenance free batteries.

    - Motion alarm.

    - Beacon light.

    - Automotive Horn.

    - Safety harness.

# Industry Standards / Requirement(s):

    - Industry Standards: Underwriters Laboratory (UL) listed or approved equivalent. Conformité Européene (CE) is not acceptable for a standard.

    - Warranty period shall be for (1) year from date of delivery against defective material and workmanship and shall include parts, service and labor.

  - Information Technology Requirements:  N / A

# Color Requirement(s):  N / A

- Electrical Requirements:

# _____ Standard U.S.A. electrical specifications:  120 volts, 60 Hertz, 1 phase.

# _____ Pacific Theater stores 110 volt / 50 HZ / 1 PH (Applies to these stores only: Atsugi, Misawa, Sagami, Sagamihara, Yokosuka, Yokota, Camp Zama, and Kanto Plains CDC. All other stores are standard U.S.A. 60 cycle voltages.)

# Minimum 6 foot cord with a NEMA Standard plug, or a manufacturer’s equivalent to comply with the equipment plug in requirements.

# NOTE: the operating range for 120 volts is 110 to 120 volts; the operating range for 220 volts is 208 to 230 volts.

# Standard Contractor Requirements:

  - Information to be provided by the Contractor to the commissary at the time of delivery:

    - Point of Contact for Service

    - Installation and Operating Instructions

    - Parts List

  - Maintenance Sustainability Requirements: Continued parts support is required for the projected life cycle of the equipment plus 5 years.

# Special Coordinating / Safety Instructions:  N / A

STORE ORDER REQUEST

STORE NAME: __________________________    DODAAC: ____________________

QUANTITY: _______